Work Paper WPSPGENRAG0002

Low Pressure Sprinkler Nozzles

Revision # 1

San Diego Gas & Electric Company

Energy Efficiency Engineering

Low Pressure Sprinkler Nozzles

Measure Code O-D1

# At-A-Glance Summary

| Applicable Measure Codes: | O-D1 |
| --- | --- |
| Measure Description: | This measure replaces a high pressure (50 psi or higher) sprinkler nozzle with a hand move low pressure sprinkler nozzle (portable or permanent solid set) |
| Energy Impact Common Units: | Per nozzle |
| Base Case Description: | Source: DEER 2005. High-pressure, sprinkler nozzle (50 psi or higher operating pressure). |
| Base Case Energy Consumption: | The base case energy consumption is based on the pumping energy required to deliver irrigation water through the baseline high-pressure sprinklers. |
| Measure Energy Consumption: | The measure energy consumption is based on the pumping energy required to deliver irrigation water through the new low-pressure sprinklers. |
| Energy Savings (Base Case – Measure) | Source: DEER 2005 - D03-970 and D03-971. The energy savings for climate zone 7. |
| Costs Common Units: | Per nozzle |
| Base Case Equipment Cost ($/unit): | Source: DEER 2005 - D03-970 and D03-971. $0.90 per nozzle |
| Measure Equipment Cost ($/unit): | Source: DEER 2005 - D03-970 and D03-971. $0.34 per portable nozzle and $.84 per solid set nozzle. Average equipment cost $.59 per nozzle |
| Measure Incremental Cost ($/unit): | ROB = $1.24 D03-970 and $1.74 D03-971. Average measure incremental cost $1.49 Source: DEER 2005 - D03-970 and D03-971. |
| Effective Useful Life (years): | Source: DEER 2008.D03-970 3 years and D03-971 5 years. Average EUL 4 years |

AtAt A Glance Measure List

| Measure Code | DEER Run ID | Measure Description | Building Type | Building Vintage | Climate Zone | Peak Electric Demand Reduction (kW/nozzle) | Annual Electric Savings (kWh/ nozzle) | Annual Gas Savings (therms/nozzle) | Base Case Cost ($/nozzle) | Measure Cost ($/nozzle) | IMC ($/nozzle) | Effective Useful Life (years) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CFRM07AVLPSHd | Low Pressure Sprinkler Nozzle - Portable | ANY | ANY | Z07 | 0.008 | 8 | 0 | $0.90 | $0.34 | $1.24 | 3 |
|  | CFRM07AVLPSHd | Low Pressure Sprinkler Nozzle - Solid set | ANY | ANY | Z07 | 0.003 | 7 | 0 | $0.90 | $0.84 | $1.74 | 5 |
| O-D1 |  | Low Pressure Sprinkler Nozzle Portable or Solid Set | ANY | ANY | Z07 | .006 | 7.5 | 0 | $0.90 | $0.59 | $1.49 | 4 |

# Work Paper Approvals

| Peter Ford Manager, Energy Efficiency Engineering | Date |
| --- | --- |
| Charles Harmstead Supervisor, Energy Efficiency Engineering | Date |

# Document Revision History

Revision # 		Date 		         Description		         Author (Company)

| Revision 1 | 10/8/13 | Adapted from PGECOAGR112 R3.doc dated 8/28/12. Revised for CZ7 with portable and solid set sprinklers combined to form one Product code (measure) | Charles Harmstead |
| --- | --- | --- | --- |

# Table of Contents

At-A-Glance Summary	i

At-A-Glance Measure List	ii

Work Paper Approvals	iv

Document Revision History	v

Table of Contents	vi

List of Tables	vii

List of Figures	vii

Section 1. General Measure & Baseline Data	1

1.1 Measure Description & Background	1

1.2 DEER Differences Analysis	2

1.3 Codes & Standards Requirements Analysis	2

1.4 EM&V, Market Potential, and Other Studies	2

1.5 Base Cases for Savings Estimates: Existing & Above Code	2

1.6 Base Cases & Measure Effective Useful Lives	3

1.7 Net-to-Gross Ratios for Different Program Strategies	3

Section 2. Calculation Methods	3

2.1 Electric Energy Savings Estimation Methodologies	3

2.2. Demand Reduction Estimation Methodologies	3

2.3. Gas Energy Savings Estimation Methodologies	3

Section 3. Load Shapes	3

3.1 Base Case Load Shapes	4

3.2 Measure Load Shapes	4

Section 4. Base Case & Measure Costs	4

4.1 Base Case Costs	4

4.2 Measure Costs	4

4.3 Incremental & Full Measure Costs	4

Index	5

References	6

# List of Tables

Table-1:  Net-to-Gross Ratios	3

# List of Figures

There are no figures in this work page

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Catalog Description

LOW PRESSURE SPRINKLER NOZZLES

Requirements:

- Customer must have electricity distributed by PG&E to the installation address.

- Must convert from a high-pressure, sprinkler system nozzle (50 psi operating pressure or more at the sprinkler head) to a low-pressure sprinkler system nozzle.

  - Low-pressure sprinkler system nozzles must replace high-pressure sprinkler system nozzles one-for-one.

- A pumping plant analysis must be completed after the replacement to ensure reasonable pumping efficiency.

  - Must have 45% Overall Pumping Efficiency or above.

- Hand move or Permanent (solid set) systems apply.

| Product Code | Description |
| --- | --- |
| O-D1 | Low Pressure Sprinkler Nozzles, Hand Move or Permanent (Solid Set) |

Program Restrictions and Guidelines

Terms and Conditions

- Customer must have electricity distributed by PG&E to the installation address

- Must convert from a high-pressure, sprinkler system nozzle (50 psi operating pressure or more at the sprinkler head) to a low-pressure sprinkler system nozzle

  - Low-pressure sprinkler system nozzles must replace high-pressure sprinkler system nozzles one-for-one

- A pumping plant analysis must be completed after the replacement to ensure reasonable pumping efficiency

  - Must have 45% Overall Pumping Efficiency or above

- Micro/Mini sprinklers qualify only when there is a one for one change out (therefore the reason for having the restriction). Usually a standard field sprinkler covers an area of 30 x 30 feet or 40 x 60 feet and when changed to a micro/mini sprinklers the volume of the micro/mini sprinklers increase to cover the same area so the savings go down.

- Our catalog requires a 45% OPE vs. 55% OPE stated in DEER 2005, since the 55% is an average number of pumps tested in the field therefore some pumps have lower OPE than 55% and some pumps have higher OPE than 55%, so in order to capture most of the savings we use a lower limit for our program qualification. The intent of the OPE restriction is to: 1-Make sure the customers are aware of the potential impact on OPE from a change out and 2-To make sure we don’t significantly deteriorate OPE.

Market Applicability

This measure is applicable to sprinkler irrigated farmland in the SDG&E service territory that relies on electric pumping to water crops.

Technical Description

A sprinkler nozzle distributes water over irrigation fields.  With high pressure nozzles, water droplets break up and distribute in different sizes, insuring that an area has equal amount of water distribution.  Low pressure nozzles utilize different orifice shapes to accomplish the same water distribution at lower pressure and energy. A hand move (portable) system is movable throughout the area being sprinkled. A solid set system is fixed in the area being irrigated. Savings come from the reduction in energy needed to deliver water at the sprinkler heads at the lower pressure.

## 1.2 DEER Differences Analysis

This workpaper matches with the DEER 2005 low pressure sprinkler nozzle measure – solid set and portable, DEER Measure ID - D03-970 and D03-971. No change was made to the DEER analysis. DEER does not provide savings information for climate zone 10. Therefore; to be conservative, the saving numbers for climate zone 10 is taken from the lowest savings provided in DEER 2005 (climate zone 7).

This measure was not included in the DEER 2011 update; however, DEER 2005 still represents the best data currently available.  As a result, DEER 2005 values will continue to be used until better data is available.

## 1.3 Codes & Standards Requirements Analysis

Title 20: These measures do not fall under Title 20 of the California Energy Regulations.

Title 24: These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards: These measures do not fall under Federal DOE or EPA Energy Regulations.

Reducing the water flow will reduce the need for runoff consideration. While California State Water Code requires landowners to obtain a permit or conditional waiver for runoff, there is no code requirement for installation of low-pressure sprinkler nozzles.

## 1.4 EM&V, Market Potential, and Other Studies

There were no EM&V studies identified that address low-pressure sprinklers. DEER 2005 has the most relevant information.

## 1.5 Base Cases for Savings Estimates: Existing & Above Code

The base case assumes a high-pressure nozzle that has at least 50 psi operating pressure or more at the sprinkler nozzles.

## 1.6 Base Cases & Measure Effective Useful Lives

The effective useful life is 3 years for the portable sprinkler nozzle and 5 years for the permanent or solid set type sprinkler nozzle. Based on DEER 2008Error: Reference source not found. Average Measure life of 4 years is used.

## 1.7 Net-to-Gross Ratios for Different Program Strategies

Table-1 below shows Net-to-Gross ratio for this measureError: Reference source not found.  The value of 0.6 comes from the DEER 2011 update and represents the default NTG for “All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years”, specifically for the Agricultural sector.

Table-1:  Net-to-Gross Ratios

|  |  | DEER Spreadsheet |  |
| --- | --- | --- | --- |
| Program Approach | NTG | File name | Cell Number |
| ROB | 0.60 | DEER2011-NTG_IncludingCarryoversFromDEER2008_2011-12-07.xls | T150 |

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

Electric savings in this measure are taken from the DEER Measure ID - D03-970 and D03-971. These savings are presented in kWh/nozzle. The savings are the difference in annual energy consumed between the high pressure nozzle system and low pressure estimates.  DEER has provided savings numbers for climate zone 7. For SDG&E climate zone10 the more conservative saving number from climate zone 7 is used.

## 2.2. Demand Reduction Estimation Methodologies

Demand reduction estimations in this measure are taken directly from the DEER Measure ID - D03-970 and D03-971 where they are presented in kW/nozzle. The savings is the difference in power demand between the high pressure nozzles to low pressure nozzle. DEER has provided saving numbers for climate zone 7. For SDG&E climate zone10 the more conservative saving number from climate zone 7 is used.

## 2.3. Gas Energy Savings Estimation Methodologies

There is no gas saving associated with this measure.

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

## 3.1 Base Case Load Shapes

The base case load shape would be expected to follow a typical agricultural pumping end use load shape.

## 3.2 Measure Load Shapes

For purposes of the net benefits estimates in the E3 calculator, what is required is the load shape that ideally represents the difference between the base equipment and the installed energy efficiency measure.  This difference load profile is what is called the Measure Load Shape and would be the preferred load shape for use in the net benefits calculations.

The measure load shape for this measure is determined by the E3 calculator based on the applicable agricultural market sector and the pumping end-use.

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Costs

There is no update on cost information in DEER 2011. The base cost is $0.90 per nozzle for either portable or permanent nozzles. Based on DEER 2005 database, measure ID - D03-970 and D03-971.

## 4.2 Measure Costs

The equipment cost is $0.34 and $0.84 per nozzle for portable (hand move) and permanent (solid set) nozzles respectively. Based on DEER 2005 database measure ID - D03-970 and D03-971. An average equipment cost of $.59 is used.

## 4.3 Incremental & Full Measure Costs

The measure is a replace on burnout (ROB) measure where full costs are applicable. DEER 2005 Measure ID - D03-970 and D03-971 provide an installed cost (labor cost of replacement plus materials) for the portable (hand move) nozzles at $1.24 per nozzle and for the solid set (permanent) nozzle at $1.74.  An average incremental cost of $1.49 is used for this Product code (measure).

# Index

climate zones	3

Costs	4

DEER	i, ii, iii, 2, 3, 4

effective useful life	3

Load Shapes	3, 4

operating pressure	i, 2

runoff	2

# References

-DEER 2008, http://www.deeresources.com/deer0911planning/downloads/EUL_Summary_10-1-08.xls

-Central Valley Regional Water Quality Control Board, Fact Sheet of Conditional Waiver of Waste Discharge Requirements for Discharges from Irrigated Land, July 2006. http://www.waterboards.ca.gov/centralvalley/water_issues/irrigated_lands/general_prog_info/irrlands_disch_fact_sht.pdf

- “2004-2005 Database for Energy Efficiency Resources (DEER) update Study, final Report” prepared for Southern California Edison, prepared by Itron, Inc., December 2005, Measure ID – D03970 and D03-971.