Work Paper WPSDGENRCC0006

Revision 3

San Diego Gas & Electric

Energy Efficiency Engineering

Commercial Convection Oven-Electric and Gas

### Core Measure Summary Table

| General Measure Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PT |  | 1st Baseline Period |  |  |  | 2nd Baseline Period |  |  |  | TOU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Name | PreDesc | StdDesc | Measure RunID | Solution Code | CZ | Building Type | UseCategory | UseSubCategory | TechGroup | TechType | PreTechGroup | PreTechType | StdTechGroup | StdTechType | Load Shape | EUL_ID | EUL | Unit Definition | Program Type (NEW, ROB, RET) | Applicable Code | Gross Unit Annual Electricity Savings (kWh/unit) | User Entered kW Savings per unit (kW/unit) | Gas Savings (Therms) | 1st Baseline Useful Life | Gross Unit Annual Electricity Savings (kWh/unit) | User Entered kW Savings per unit (kW/unit) | Gas Savings (Therms) | 2nd Baseline Useful Life | % Eligible for TOU AC Adjustment |
| Commercial Half-Size Convection Oven (Electric) |  | Convection Oven - Electric (65% efficiency) | N/A | FS-G1 | 07 | Restaurant - Sit-Down | FoodServ | Cooking | Cook_equip | OvenConv |  |  | Cook_equip | OvenConv | 35-OTI-OtherIndustrial-PROC_OTH | Cook-ElecConvOven | 12.0 | Unit | ROB | No | 2518.00 | 0.60 | 0.00 | 12.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Commercial Full-Size Convection Oven (Electric) |  | Convection Oven - Electric (65% efficiency) | N/A | FS-G1 | 07 | Restaurant - Sit-Down | FoodServ | Cooking | Cook_equip | OvenConv |  |  | Cook_equip | OvenConv | 35-OTI-OtherIndustrial-PROC_OTH | Cook-ElecConvOven | 12.0 | Unit | ROB | No | 2774.00 | 0.60 | 0.00 | 12.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Commercial Large Full-Size Convection Oven (Electric) |  | Convection Oven - Electric (65% efficiency) | N/A | FS-G1 | 07 | Restaurant - Sit-Down | FoodServ | Cooking | Cook_equip | OvenConv |  |  | Cook_equip | OvenConv | 35-OTI-OtherIndustrial-PROC_OTH | Cook-ElecConvOven | 12.0 | Unit | ROB | No | 3613.00 | 0.80 | 0.00 | 12.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Commercial Half-Size Convection Oven (Gas) |  | Convection Oven - Gas (30% efficiency) | N/A | FS-G2 | 07 | Restaurant - Sit-Down | FoodServ | Cooking | Cook_equip | OvenConv |  |  | Cook_equip | OvenConv | 35-OTI-OtherIndustrial-PROC_OTH | Cook-GasConvOven | 12.0 | Unit | ROB | No | 0.00 | 0.00 | 226.00 | 12.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Commercial Full-Size Convection Oven (Gas) |  | Convection Oven - Gas (30% efficiency) | N/A | FS-G2 | 07 | Restaurant - Sit-Down | FoodServ | Cooking | Cook_equip | OvenConv |  |  | Cook_equip | OvenConv | 35-OTI-OtherIndustrial-PROC_OTH | Cook-GasConvOven | 12.0 | Unit | ROB | No | 0.00 | 0.00 | 357.00 | 12.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Commercial Large Full-Size Convection Oven (Gas) |  | Convection Oven - Gas (30% efficiency) | N/A | FS-G2 | 07 | Restaurant - Sit-Down | FoodServ | Cooking | Cook_equip | OvenConv |  |  | Cook_equip | OvenConv | 35-OTI-OtherIndustrial-PROC_OTH | Cook-GasConvOven | 12.0 | Unit | ROB | No | 0.00 | 0.00 | 456.00 | 12.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Note: For the complete list of Measures, refer to the attached calculation spreadsheet []

### Costing and NTG Summary Table

| General Measure Information |  |  |  | PT |  | NTG |  |  | IR | 1st Baseline Period | 2nd Baseline Period | IMC | DIM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Name | Solution Code | CZ | Unit Definition | Program Type (NEW, ROB, RET) | Applicable Code | NTG Non-Res. | NTG Res. | NTG Multi Family | Installation Rate | Gross Measure Cost per unit | Gross Measure Cost per unit | Incremental Measure Cost per unit | Delivery & Incentive Method |
| Commercial Half-Size Convection Oven (Electric) | FS-G1 | 7 | Unit | ROB | No | 0.60 |  |  | 1.00 | $793.00 | $0.00 | $793.00 | Financial Support / Down-Stream Incentive - Deemed |
| Commercial Full-Size Convection Oven (Electric) | FS-G1 | 7 | Unit | ROB | No | 0.60 |  |  | 1.00 | $1,007.00 | $0.00 | $1,007.00 | Financial Support / Down-Stream Incentive - Deemed |
| Commercial Large Full-Size Convection Oven (Electric) | FS-G1 | 7 | Unit | ROB | No | 0.60 |  |  | 1.00 | $649.00 | $0.00 | $649.00 | Financial Support / Down-Stream Incentive - Deemed |
| Commercial Half-Size Convection Oven (Gas) | FS-G2 | 7 | Unit | ROB | No | 0.60 |  |  | 1.00 | $490.00 | $0.00 | $490.00 | Financial Support / Down-Stream Incentive - Deemed |
| Commercial Full-Size Convection Oven (Gas) | FS-G2 | 7 | Unit | ROB | No | 0.60 |  |  | 1.00 | $1,286.00 | $0.00 | $1,286.00 | Financial Support / Down-Stream Incentive - Deemed |
| Commercial Large Full-Size Convection Oven (Gas) | FS-G2 | 7 | Unit | ROB | No | 0.60 |  |  | 1.00 | $1,421.00 | $0.00 | $1,421.00 | Financial Support / Down-Stream Incentive - Deemed |

Note: For the complete list of Measures, refer to the attached calculation spreadsheet [A]

# Document Revision History

| Revision # | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 12/12/2007 | David Zabrowski / Fisher-Nickel, Inc. | Original work paper: Commercial Convection Oven PGECOFST101 R0.doc |
| 1 | 07/17/2010 | Lucie Sidibe / SDG&E | Adopted from PGECOFST101, Revision 2*refer to attachment for original work paper Summary of changes: 1-Work Paper run ID was changed to mirror SDGE cataloging needs. 2.Measure code was updated to meet SDGE Program needs |
| 2 | 06/15/2012 | Max Twogood / SDG&E | Revised from Commercial Convection Ovens PGECOFST101 R4 -Revised June 7 2012.doc dated June 7, 2012. |
| 3 | 06/23/2014 | Judelson Enriquez / RMS Energy Consulting, LLC | 1. Updated to new template. 2. Generated calculation spreadsheet based on IOU statewide Calculation Template, with additional columns for Mark M. and removed CZ cost factors. 3. Incorporated PG&E’s PGECOFST101 R5Commercial Convection Ovens.doc WP revisions, dated April 21, 2014. 4. Savings values were revised, as well as cost for large full-size conv. oven. |

# Section 1. General Measure & Baseline Data

## 1.1  Measure & Delivery Description

### 1.1a Measure Description

This work paper documents the rationale or the Energy Efficient Commercial Convection Oven (Electric and Gas) measure as listed in the Commercial Food Service Catalog [D].

Table 1 Measure Name

| Product Code | Measure name |
| --- | --- |
| FS-G1 | Commercial Convection Oven – Electric |
| FS-G2 | Commercial Convection Oven – Gas |

Catalog Descriptions

FS-G1:

- The half-size electric oven must meet ENERGY STAR® specifications or have a tested heavy load potato cooking energy efficiency ≥ 71% and idle rate ≤ 1.0 kW utilizing ASTM Standard F1496 [].

- The full-size (≤ 5 pans) electric oven must meet ENERGY STAR® specifications or have a tested heavy load potato cooking energy efficiency ≥ 71% and idle rate ≤ 1.6 kW utilizing ASTM Standard F1496

- The large full-size (> 5 pans) electric oven must have a tested heavy load potato cooking energy efficiency ≥ 73% and idle rate ≤ 1.9 kW utilizing ASTM Standard F1496

FS-G2:

- The half-size gas oven must have a tested heavy load potato cooking energy efficiency ≥ 46% and idle rate ≤ 8,000 Btu/h utilizing ASTM Standard F1496

- The full-size (≤ 5 pans) gas oven must meet ENERGY STAR® specifications or have a tested heavy load cooking energy efficiency ≥ 46% and idle rate ≤ 12,000 Btu/h utilizing ASTM Standard F1496

- The large full-size (> 5 pans) gas oven must have a tested heavy load potato cooking energy efficiency ≥ 46% and idle rate ≤ 13,000 Btu/h utilizing ASTM Standard F1496

Product Technical Description

Table 2 DEER Use and Technology Table

Commercial convection ovens are the most widely used appliances in the food service industry. Many food service operations rely heavily on the versatility of ovens. Operators can cook varieties of foods in large quantities with a single appliance. An oven can be simply described as a fully enclosed, insulated chamber used to heat food. With competition rising among equipment manufacturers, new designs that incorporate timesaving features via sophisticated control packages are being introduced.

Ovens represent the largest appliance category in terms of the types of units manufactured of any of the major cooking equipment categories. This versatility and diversity mean that they can be found in almost any type of food service operation. A recent US study showed that 95% of commercial (non-institutional) operations reported using at least one type of oven; 98% of noncommercial (institutional) operations reported the same. The percentage of operations, commercial and institutional, using general bake ovens was 52% and 56%, respectively. Fifty percent of the operations in the commercial sector reported using convection ovens as compared to 83% of noncommercial operations [].

Convection oven performance is determined by applying the ASTM Standard Test Method for the Performance of Convection Ovens (F1496). The ASTM standard test method is considered to be the industry standard for quantifying the energy consumption, efficiency and cooking performance of convection ovens.

In 2013, ASTM F1496 was updated to accommodate a wider range of oven cavity sizes. The Energy Star v2.1 specification for commercial ovens now references the updated ASTM standard in addition to tightening the idle rate requirements for gas full-size convection ovens.

### 1.1b Delivery and Incentive Mechanism

The delivery method for the measure(s) in this workpaper is:

- Financial Support – Downstream Incentive – Deemed

Since there are no EM&V studies on the useful life of convection ovens and it is standard practice in the commercial foodservice industry to purchase equipment only when it is needed (e.g. replacement or additional capacity), the measure install types are:

- Replace on Burnout (ROB) or New Construction (NC)

### 1.1c Measure Requirements

For Energy Efficiency deemed measures, measure eligibility requirements are established in the San Diego Gas & Electric Energy Efficiency Business Rebates Product Catalog [].

This measure includes new commercial electric or gas convection ovens that are ENERGY STAR® qualified [] or meet the qualifications listed in Table 3. ENERGY STAR® maintains an updated list of qualifying products and specifications at www.energystar.gov. Consult with the manufacturer or manufacturer’s representative to determine if a non-ENERGY STAR® qualified model meets the efficiency requirements in Table 3. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance has a cooking-energy efficiency that meets the requirements. The rebate is downstream provided to the customer as the time of sale upon receipt of application and invoice. This is not a direct install program.

Table 3 Energy Efficiency Requirements for Commercial Convection Ovens.

| Measure Code | Convection oven Type | Cooking-Energy Efficiency* | Idle Energy Rate |
| --- | --- | --- | --- |
| F187 | Electric Half-Size Convection Ovens | ≥ 71% | ≤ 1.0 kW |
|  | Electric Full-Size (≤ 5 pans) Convection Ovens | ≥ 71% | ≤ 1.6 kW |
|  | Electric Full-Size (> 5 pans) Convection Ovens | ≥ 73% | ≤ 1.9 kW |
| F188 | Gas Half-Size Convection Ovens | ≥ 46% | ≤ 8,000 Btu/h |
|  | Gas Full-Size (≤ 5 pans) Convection Ovens | ≥ 46% | ≤ 13,000 Btu/h |
|  | Gas Full-Size (> 5 pans) Convection Ovens | ≥ 46% | ≤ 13,000 Btu/h |

*Based on the heavy-load potato test in ASTM F1496.

Product Base Case and Measure Case Data

This Work Paper uses ASTM Standard Test Method for the Performance of Convection Ovens (F1496) for calculation of energy use and demand, based on testing in an approved and qualified laboratory. In the absence of mandatory regulations for testing commercial ovens, there is little incentive on the part of equipment manufacturers to have their baseline equipment tested. Therefore, the ASTM performance parameters for baseline equipment were drawn from a sample of economy grade equipment tested by the Food Service Technology Center and is summarized in Table 4.

Table 4 Baseline ASTM test results for Commercial Convection Ovens.

| Convection Oven Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Half-Size | 1,500 W | 65% | 45 lb/h |
| Electric Full-Size | 2,000 W | 65% | 70 lb/h |
| Electric Large Full-Size | 2,500 W | 65% | 100 lb/h |
| Gas Half-Size | 12,000 Btu/h | 30% | 45 lb/h |
| Gas Full-Size | 18,000 Btu/h | 30% | 70 lb/h |
| Gas Large Full-Size | 21,000 Btu/h | 30% | 100 lb/h |

*Based on the Heavy-load potato test in ASTM F1496. [B].

The measure case data was drawn from the list of commercial convection ovens that have been tested by The PG&E Food Service Technology Center in San Ramon, the Southern California Gas Company Foodservice Equipment Center in Downey and the Southern California Edison Foodservice Technology Center in Irwindale. The complete list of energy-efficient convection ovens as of May 30, 2012 is in Appendix A and the averages are summarized in Table 5.

Table 5 Measure ASTM test results for Commercial Convection Ovens.

| Convection Oven Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Half-Size | 880 W | 72% | 53 lb/h |
| Electric Full-Size | 1,400 W | 73% | 82 lb/h |
| Electric Large Full-Size | 1,730 W | 76% | 137 lb/h |
| Gas Half-Size | 8,500 Btu/h | 45% | 83 lb/h |
| Gas Full-Size | 10,870 Btu/h | 48% | 81 lb/h |
| Gas Large Full-Size | <13,000Btu/h | 46% | 100 lb/h |

* Based on the Heavy-load potato test in ASTM F1496 [B].

The Food Service Technology Center conducted an assessment of major commercial cooking appliance technologies in 2002, which included a chapter on ovens [].  The study showed that oven efficiency varies from 10% to 80%, based on primary fuel source and oven type (convection, combination, conventional, etc.). A further Food Service Technology Center study included convection oven performance data, which suggests a baseline efficiency of 65% for electric convection ovens and 30% gas convection ovens, respectively [].

Since the current Title 20 regulations do not include a minimum performance requirement for convection ovens, the base case for existing models of electric convection ovens was determined from Table 3-2 in the Food Service Technology Center study [C].

Market Applicability

This measure is applicable to any commercial cooking application, including (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

## 1.2  DEER Differences Analysis

The 2014 DEER database does not contain information on energy use or savings for an energy-efficient electric or gas convection oven measure. The only reference in DEER for Commercial cooking equipment is for Estimated Useful Life.

Table 6 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER 2014 does not contain these measures. |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

## 1.3  Code Analysis

This measure is not governed by either state or federal codes and standards.

ASTM Standard Test Method for the Performance of Convection ovens (F1496) is applicable for estimating energy use and cooking performance. It was used to estimate the energy consumption of the base case and measure equipment.

The 2014 California Title 20 Appliance Efficiency Regulations include standards for both federally regulated appliances and nonfederally-regulated appliances that are sold or offered for sale in California; however, Title 20 does not govern the measures found in this workpaper. California Title 20 does require reporting the convection oven idle energy rate per ASTM F1496 for the CEC Appliance Database, but the standard contains no minimum performance requirement [].

The 2013 California Title 24 Building Energy Efficiency Standards include standards to improve the energy efficiency of newly constructed building and additions and alterations to existing buildings for residential and nonresidential buildings; however, Title 24 does not govern the measures found in this workpaper.

Table 7 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2014) | N/A | N/A |
| Title 24 (2013) | N/A | N/A |
| Federal Standards | N/A | N/A |

## 1.4  Measure Effective Useful Life

To obtain the EUL value, the updated CPUC ED EUL table documentation issued on February 5, 2014, “DEER2014-EUL-table-update_2014-02-05.xlsx”, was consulted. Table 8 below identifies the value/methodology used for the measures in this work paper.

Table 8 DEER EUL Value/Methodology

| Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- |
| Non-Residential | Food Service | Convection Oven | 12 | 4 |

Table 9 identifies the EUL IDs used in this workpaper.

Table 9 EUL ID Summary

| EUL_ID | Description | Sector |
| --- | --- | --- |
| Cook-ElecConvOven | Convection Oven - Electric | Com |
| Cook-GasConvOven | Convection Ovens - Gas | Com |

## 1.5  Net-to-Gross Ratios for Different Program Strategies

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” [] on the DEER website as required by Version 4 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual. The relevant NTGR for this measure is shown in Table 10 below.

Table 10 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID |
| --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All |

*Denotes that the column is taken from the DEER NTG Table.

## 1.6  Time-of-Use Adjustment Factor

As directed by the CPUC in decision 06-06-063 dated June 29, 2006, time-of-use (TOU) adjustment factors are to be applied for residential A/C and commercial A/C (packaged and split-system direct-expansion cooling) measures only.   Since this is not an A/C measure, the TOU adjustment factor is zero.  Additionally, if a measure is assigned a DEER08 load shape, i.e. the load shape starts with “DEER:” the TOU assigned to that measure should also be zero.

Table 11  TOU Summary Table

| Measure | % |
| --- | --- |
| Commercial Convection Oven | 0 |

*Note:  Check Section 3 if a measure appears to require a non-zero percentage but is assigned zero. If the load shape is a DEER08 load shape, a TOU of zero is correct.

# Section 2. Energy Savings & Demand Reduction Calculations

## 2.1 Electric Energy Savings Estimation Methodologies

This measure would follow the hours of operation for quick serve and full serve restaurants as noted in the ASTM standards. For this measure the annual hours of operation are considered 4380.

12 hrs/day * 365 day/yr = 4380

The industry standard for energy use and cooking performance of convection ovens is ASTM Standard Test Method for the Performance of Convection ovens (F1496). Table 12 - Table 14 show examples of the calculation results for electric convection ovens based on ASTM F1496 test results.

Table 12 Commercial Electric Half-Size Convection Oven Cost-Effectiveness Example

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.00 | 0.90 |
| Idle Energy Rate (kW) | 1.5 | 0.88 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 72% |
| Production Capacity (lbs/hr) | 45 | 53 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 26.6 | 19.7 |
| Average Demand (kW) | 2.2 | 1.6 |
| Estimated Demand Reduction (kW) | - | 0.6 |
| Annual Energy Consumption (kWh) | 9,709 | 7,191 |
| Estimated Energy Savings (kWh/yr) | - | 2,518 |
| Annual Energy Cost ($) | $1,262 | $935 |
| Estimated Cost Savings ($/yr) | - | $327 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL) | See Section 1.4 | See Section 1.4 |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

Table 13 Commercial Electric Full-Size Convection Oven Cost-Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.50 | 1.00 |
| Idle Energy Rate (kW) | 2.00 | 1.40 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 73% |
| Production Capacity (lbs/hr) | 70 | 82 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 33.4 | 25.8 |
| Average Demand (kW) | 2.8 | 2.2 |
| Estimated Demand Reduction (kW) | - | 0.6 |
| Annual Energy Consumption (kWh) | 12,191 | 9,417 |
| Estimated Energy Savings (kWh/yr) | - | 2,774 |
| Annual Energy Cost ($) | $1,585 | $1,224 |
| Estimated Cost Savings ($/yr) | - | $361 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)b | See Section 1.4 | See Section 1.4 |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

Table 14 Commercial Electric Large Full-Size Convection Oven Cost-Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.70 | 1.20 |
| Idle Energy Rate (kW) | 2.50 | 1.73 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 76% |
| Production Capacity (lbs/hr) | 100 | 137 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 39.8 | 29.9 |
| Average Demand (kW) | 3.3 | 2.5 |
| Estimated Demand Reduction (kW) | - | 0.7 |
| Annual Energy Consumption (kWh) | 14,527 | 10,914 |
| Estimated Energy Savings (kWh/yr) | - | 3,613 |
| Annual Energy Cost ($) | $1,889 | $1,419 |
| Estimated Cost Savings ($/yr) | - | $470 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)b | See Section 1.4 | See Section 1.4 |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

Daily Energy Consumption Calculation and Definitions

Daily Energy Consumption Calculation and Definitions

EDAY = (LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)] 
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (kWh/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking based on ASTM F1496 |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (kW) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (kWh) |

## 2.2 Demand Reduction Estimation Methodologies

An oven’s actual contribution to a building’s peak demand may vary significantly depending on its usage pattern in relation to that of other electric equipment in the facility (operating schedule, appliance on time, etc.). The probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance. Therefore, it has been assumed that the probable contribution to the building’s peak demand is equal to the appliance’s average demand.

The demand reduction estimation is based on measured data for standard efficiency electric convection ovens and for high-efficiency convection ovens. The measured data are derived from tests conducted under ASTM Standard Test Method for the Performance of Convection ovens (F1496).

ASTM F1496 provides standard conditions under which convection oven energy use is measured. The estimated demand reduction of 580 Watts for a Half-Size oven, 630 Watts for a Full-Size oven and 830 Watts for a Large Full-Size oven is based on data from tests of standard efficiency and high efficiency convection ovens. Applying a Coincidence Factor of 0.9 per the DEER methodology [] yields a Demand Savings of 520 Watts for a Half-Size oven, 570 Watts for a Full-Size oven and 750 Watts for a Large Full-Size oven.

## 2.3 Gas Energy Savings Estimation Methodologies

The industry standard for energy use and cooking performance of convection ovens is ASTM Standard Test Method for the Performance of Convection ovens (F1496). Table 15 - Table 17 show examples of the calculation results under ASTM F1496.

Table 15 Commercial Gas Half-Size Convection Oven Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 13,000 | 7,500 |
| Idle Energy Rate (Btu/hr) | 12,000 | 8,500 |
| Cooking-Energy Efficiency (%) | 30% | 45% |
| Production Capacity (lb/hr) | 45 | 55 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 250 | 250 |
| Daily Energy Consumption (Btu) | 210,667 | 148,739 |
| Annual Energy Consumption (therms)a | 769 | 543 |
| Estimated Energy Savings (therms/yr) | - | 226 |
| Annual Energy Cost ($) | $769 | $543 |
| Estimated Cost Savings ($/yr) | - | $226 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL) | See Section 1.4 | See Section 1.4 |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

Table 16 Commercial Gas Full-Size Convection Oven Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 19,000 | 10,094 |
| Idle Energy Rate (Btu/hr) | 18,000 | 10,870 |
| Cooking-Energy Efficiency (%) | 30% | 48% |
| Production Capacity (lb/hr) | 70 | 81 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 250 | 250 |
| Daily Energy Consumption (Btu) | 288,119 | 176,480 |
| Annual Energy Consumption (therms)a | 1,052 | 644 |
| Estimated Energy Savings (therms/yr) | - | 408 |
| Annual Energy Cost ($) | $1,052 | $644 |
| Estimated Cost Savings ($/yr) | - | $408 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL) | See Section 1.4 | See Section 1.4 |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

Table 17 Commercial Gas Large Full-Size Convection Oven Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 21,000 | 11,000 |
| Idle Energy Rate (Btu/hr) | 21,000 | 13,000 |
| Cooking-Energy Efficiency (%) | 30% | 46% |
| Production Capacity (lb/hr) | 100 | 100 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 250 | 250 |
| Daily Energy Consumption (Btu) | 330,083 | 205,098 |
| Annual Energy Consumption (therms)a | 1,205 | 749 |
| Estimated Energy Savings (therms/yr) | - | 456 |
| Annual Energy Cost ($) | $1,205 | $749 |
| Estimated Cost Savings ($/yr) | - | $456 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL) | See Section 1.4 | See Section 1.4 |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

Daily Energy Consumption Calculation and Definitions

EDAY = (LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)] 
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (Btu/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (Btu/lb) = Btu/pound of energy absorbed by food product during cooking based on ASTM F1496 |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (Btu/h) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (Btu) |

For all the savings discussed above, there is an installation rate applied to values associated with the installation GSIA ID in Table 18. The GSIA ID is identified in the ex-ante implementation tables for all programs and measures. The installation rate (IR) is applied to the gross savings calculations using the values associated with the IDs below.

Table 18 Gross Savings Installation Adjustment (GSIA) IDs

| GSIA_ID | Description | Sector | BldgType | UseCategory | TechType |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA | Any | Any | Any | Any |

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio for electricity.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

Base Cases Load Shapes

The base case would be expected to follow a typical nonresidential foodservice end use load shape.

Commercial convection oven load shapes differ among food service facilities (quick service, casual dining, hotels, college, schools, hospitals etc) depending on daily menu variations, hours of operation, serving periods, day-of-week, and facility location (city downtown, suburban mall, access to interstate highways, etc.). Consequently, applicable average TOU and hourly load shapes for convection ovens are unavailable. The ASTM Standard Test Method used to generate energy use data is based on hours of use and operating state (preheat, idle, and heavy-load cooking). Generally, convection ovens are used to prepare food within a few days to a few hours before it is served, so loads tend to not necessarily be coincident with regular meal periods (breakfast, lunch, and dinner). Between meal periods convection ovens may be used to prepare ingredients for either the next meal period or for menu items to be served the next several days (in which case the ingredients are refrigerated immediately after cooking).

Measure Load Shapes

For purposes of the net benefits estimates in the E3 calculator, what is required is the load shape that ideally represents the difference between the base equipment and the installed energy efficiency measure.  This difference load profile is what is called the Measure Load Shape and would be the preferred load shape for use in the net benefits calculations.

The measure load shape for the electric measure is determined by the E3 calculator based on the applicable nonresidential market sector and the foodservice end-use.

The electric demand profile for the high-efficiency electric convection oven is expected to be the same as the Base Case. The profile will vary. The Measure Load Shape for the high-efficiency convection oven will use less energy and have a slightly lower demand profile.

The gas load profile for the high efficiency gas convection oven is expected to be the same as the Base Case. The profile will vary. The Measure Load Shape for the high efficiency convection oven will use less energy.

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available. Therefore, the closest load shape chosen for this measure is the 35-OTI-OtherIndustrial-PROC_OTH load shape. See Table 19 for a list of all Building Types and Load Shapes. See the KEMA report for a more thorough discussion regarding the load shapes for this measure.

Table 19 Building Types and Load Shapes

| Building Type | Load Shape |
| --- | --- |
| Any | 35-OTI-OtherIndustrial-PROC_OTH |

# Section 4. Base Case & Measure Costs

High-efficiency convection ovens typically have a higher list price than standard efficiency convection ovens. However, high-efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high-efficiency convection ovens exhibit better baking uniformity and higher production rates that increase their cost-effectiveness.

Equipment prices for these work papers were compiled from a number of sources including, Autoquotes (autoquotes.com), equipment sales reps and manufacturer sources. Since equipment pricing in food service is closely held information and prices vary widely according to buying volume and other factors, we cannot list the sources for prices specifically.

## 4.1 Base Case Cost

The Base Case costs include only the equipment. High efficiency convection ovens require no additional labor or maintenance compared to base case convection ovens. Since this measure is applicable for ROB and NC installations, the installation and maintenance costs are expected to be the same for the customer. The estimated equipment cost is based on recent list cost data for electric and gas convection ovens and applying an industry-standard 50% discount to the manufacturer published list prices.

## 4.2 Gross Measure Cost

The Measure costs include only the equipment, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data and applying an industry-standard 50% discount to the manufacturer published list prices (see Appendix B).

## 4.3 Incremental Measure Cost

Incremental measure costs are used in the analysis.

# Appendix A

ASTM Qualifying Equipment Data for Electric Convection Ovens

| Company | Model Number | Size | Energy Efficiency (%) | Idle Energy Rate (Btu/h or kW) | Production Capacity (lbs/h) |
| --- | --- | --- | --- | --- | --- |
| APW Wyott | BCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Bakers Pride | BCO-E | Full-Size | 72% | 1.46 | 71.5 |
| BKI | COM-E | Full-Size | 72% | 1.46 | 71.5 |
| Blodgett | CTB | Half-Size | 72% | 1.01 | 52 |
| Blodgett | HV-100E | Full-Size | 70% | 1.45 | 92.7 |
| Blodgett | Mark-V | Full-Size | 74% | 1.46 | 98.6 |
| Blodgett | SHO-E | Full-Size | 73% | 1.6 | 90 |
| Blodgett | XL-50E | Half-Size | 74% | 0.63 | 51 |
| Blodgett | Zephaire 240E | Full-Size | 75% | 1.44 | 101 |
| Blodgett | Zephaire E | Full-Size | 71% | 1.57 | 87 |
| CPG | CPG-DCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Imperial | ICVE | Full-Size | 72% | 1.56 | 85 |
| Moffat | E32D | Full-Size | 78% | 0.98 | 82 |
| Moffat | E32MS | Full-Size | 80% | 0.8 | 77 |
| Moffat | E35MS | Large-Size | 73% | 1.55 | 120 |
| NDG | NDG-DCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Revent | 7802 | Full-Size | 75% | 1.1 | 76 |
| Revent | 7803 | Large-Size | 79% | 1.9 | 153 |
| Short Order | SO-DCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Southbend | SLES/10 | Full-Size | 75% | 1.55 | 84 |
| Star Manufacturing | ECOF-AP | Full-Size | 70% | 1.36 | 85 |
| Star Manufacturing | ECSF-ES | Full-Size | 76% | 1.6 | 96 |
| Tri-Star | TSCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Wells | M4200-3 | Half-Size | 71% | 0.99 | 55.2 |
| Average Performance |  | Half-Size | 72% | 0.88 | 53 |
| Average Performance |  | Full-Size | 73% | 1.40 | 82 |
| Average Performance |  | Large-Size | 76% | 1.725 | 137 |

ASTM Qualifying Equipment Data for Gas Convection Ovens

| Company | Model Number | Size | Energy Efficiency (%) | Idle Energy Rate (Btu/h or kW) | Production Capacity (lbs/h) |
| --- | --- | --- | --- | --- | --- |
| American Range | M-1 | Full-Size | 46% | 10842 | 101 |
| American Range | MSD | Full-Size | 46% | 11714 | 85 |
| Blodgett | DFG100 | Full-Size | 46% | 11200 | 84 |
| Blodgett | SHO-G | Full-Size | 46% | 10000 | 86 |
| Blodgett | Zephaire 240G | Full-Size | 46% | 12000 | 78 |
| Blue Seal | G32D5 | Full-Size | 47% | 12000 | 61 |
| Duke Manufacturing | E-101G | Full-Size | 53% | 10000 | 82 |
| Garland | MCO-GS-10 ESS | Full-Size | 46% | 11600 | 90 |
| Hobart | HGC5 | Full-Size | 48% | 11850 | 80 |
| Hobart | HGC5DX | Full-Size | 48% | 11097 | 90 |
| Imperial | ICVG-1 | Full-Size | 46% | 11400 | 82 |
| Lang | GCSF | Full-Size | 46% | 8530 | 97 |
| Moffat | G32D5 | Full-Size | 47% | 12000 | 61 |
| Vulcan | SG4D | Full-Size | 50% | 11360 | 89 |
| Vulcan | VC4G | Full-Size | 48% | 11850 | 80 |
| Vulcan | VC6G | Full-Size | 48% | 11097 | 90 |
| Vulcan | GCO2D | Half-Size | 53% | 4293 | 47 |
| Wolf | WKGD1 | Full-Size | 48% | 11850 | 80 |
| Average Performance |  | Half-Size | 53% | 4293 | 47 |
| Average Performance |  | Full-Size | 48% | 10870 | 81 |

# Appendix B

Equipment Cost Data for Electric Convection Ovens

| Group | Make | Model | Size | List Price ($) | Cost($)* |
| --- | --- | --- | --- | --- | --- |
| Baseline | Alto-Shaam | ASC-4E | Full-Size | $5,354 | $2,677 |
| Baseline | Doyon | JA6SL | Large Full-Size | $14,600 | $7,300 |
| Baseline | Duke | 5/9-E3V | Half-Size | $6,826 | $3,413 |
| Baseline | Duke | 613QE1* | Full-Size | $9,404 | $4,702 |
| Baseline | Duke | E101E* | Full-Size | $6,818 | $3,409 |
| Baseline | Garland | MCOE5 | Half-Size | $8,710 | $4,355 |
| Baseline | Hobart | HEC5 | Full-Size | $6,499 | $3,250 |
| Baseline | Lang | ECOH | Half-Size | $8,240 | $4,120 |
| Baseline | Market Forge | 3000 | Full-Size | $7,956 | $3,978 |
| Baseline | Market Forge | 2600HE | Full-Size | $8,788 | $4,394 |
| Baseline | Moffat | E311MS | Half-Size | $3,900 | $1,950 |
| Baseline | Montague | EK12A | Full-Size | $10,696 | $5,348 |
| Baseline | Vulcan | ECO2D | Half-Size | $8,370 | $4,185 |
| Baseline | Vulcan | VC4EC | Full-Size | $10,210 | $5,105 |
| Energy Efficient | Bakers Pride | BCO-E1 | Full-Size | $10,864 | $5,432 |
| Energy Efficient | Blodgett | CTB | Half-Size | $8,470 | $4,235 |
| Energy Efficient | Blodgett | Mark V | Full-Size | $12,775 | $6,388 |
| Energy Efficient | Blodgett | Xcel-50 | Half-Size | $9,120 | $4,560 |
| Energy Efficient | Blodgett | Zephaire E | Full-Size | $9,405 | $4,703 |
| Energy Efficient | Moffat | E35 | Full-Size | $11,915 | $5,958 |
| Energy Efficient | Moffat | E35 | Large Full-Size | $11,915 | $5,958 |
| Energy Efficient | Revent | 7803 | Large Full-Size | $19,882 | $9,941 |
| Energy Efficient | Southbend | SLES/10 | Full-Size | $6,188 | $3,094 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Cost Data for Gas Convection Ovens

| Group | Make | Model | Size | List Price ($) | Cost($)* |
| --- | --- | --- | --- | --- | --- |
| Baseline | Blodgett | DFG-50 | Half-Size | $10,155 | $5,078 |
| Baseline | Duke | E-101G | Full-Size | $6,818 | $3,409 |
| Baseline | Garland | G5L-N | Half-Size | $5,492 | $2,746 |
| Baseline | Lang | GCOF-T | Full-Size | $8,640 | $4,320 |
| Baseline | Market Forge | 2900 | Full-Size | $8,414 | $4,207 |
| Baseline | Market Forge | 2500HEC | Full-Size | $9,707 | $4,854 |
| Baseline | Montague | 115A | Full-Size | $12,566 | $6,283 |
| Baseline | Montague | R85A | Full-Size | $10,488 | $5,244 |
| Baseline | Southbend | BGS | Full-Size | $5,378 | $2,689 |
| Baseline | Southbend | GH-20SC | Half-Size | $6,037 | $3,019 |
| Baseline | US Range | SDG-1 | Full-Size | $7,567 | $3,784 |
| Baseline | Doyon | CA6G | Large Full-Size | $26,520 | $13,260 |
| Baseline | Doyon | JA6G | Large Full-Size | $18,390 | $9,195 |
| Baseline | LBC Bakery | LMO-G | Large Full-Size | $11,788 | $5,894 |
| Energy Efficient | Blodgett | DFG-100 | Full-Size | $14,015 | $7,008 |
| Energy Efficient | Blodgett | Zephaire-240G | Full-Size | $11,340 | $5,670 |
| Energy Efficient | Duke | E102-G | Full-Size | $12,718 | $6,359 |
| Energy Efficient | Garland | MCO-GS-10-ESS | Full-Size | $10,582 | $5,291 |
| Energy Efficient | Hobart | HGC20 | Half-Size | $5,786 | $2,893 |
| Energy Efficient | Imperial Range | ICV | Full-Size | $8,546 | $4,273 |
| Energy Efficient | Vulcan | GCOD | Half-Size | $10,630 | $5,315 |
| Energy Efficient | Vulcan | SG4D | Full-Size | $13,050 | $6,525 |
| Energy Efficient | Vulcan | VC4GD | Full-Size | $10,210 | $5,105 |
| Energy Efficient | Wolf | WKGD1 | Full-Size | $9,700 | $4,850 |
| Energy Efficient | Baxter | OV-300G | Large Full-Size | $13,133 | $6,567 |
| Energy Efficient | Blodgett | XR-8 | Large Full-Size | $25,940 | $12,970 |
| Energy Efficient | Revent | 739U | Large Full-Size | $26,151 | $13,076 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Incremental Cost Data for Energy Efficient Convection Ovens*

| Convection Oven Type | Baseline Unit Price | Energy Efficient Unit Price | Incremental Price Difference | Baseline Unit Cost | Energy Efficient Unit Cost | Incremental Measure Cost (IMC) |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Half-Size | $7,209 | $8,795 | $1,586 | $3,605 | $4,398 | $793 |
| Electric Full-Size | $8,216 | $10,229 | $2,014 | $4,108 | $5,115 | $1,007 |
| Electric Large Full-Size | $14,600 | $15,899 | $1,299 | $7,300 | $7,949 | $649 |
| Gas Half-Size | $7,228 | $8,208 | $980 | $3,614 | $4,104 | $490 |
| Gas Full-Size | $8,697 | $11,270 | $2,573 | $4,349 | $5,635 | $1,286 |
| Gas Large Full-Size | $18,899 | $21,741 | $2,842 | $9,450 | $10,871 | $1,421 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

# Attachments

# References

[] Attachment 1 – Calculation Template v2.2.xlsm.

[] American Society for Testing and Materials. Standard Test Method for the Performance of Convection Ovens. ASTM Designation F1496, in Annual Book of ASTM Standards, West Conshohocken, PA.

[] Fisher-Nickel, Inc., D. Fisher, et al., 2002. Commercial Cooking Appliance Technology Assessment. Food Service Technology Center Report 5011.02.26, pp. 3-1 to 3-27.

[] SDG&E’s Food Service & Refrigeration Catalog. http://www.sdge.com/rebates-finder/earn-rebates-your-improvements

[] 2009 Energy Star® Program Requirements for Commercial Ovens. http://www.energystar.gov/ia/partners/product_specs/program_reqs/Commercial_Ovens_Program_Requirements.pdf

[] Fisher-Nickel, Inc., D. Fisher, et al., 2002. Commercial Cooking Appliance Technology Assessment. Food Service Technology Center Report 5011.02.26, pp. 3-1 to 3-27.

[] PG&E, J. Blessent, 1994. Development and Application of a Uniform Testing Procedure for Convection Ovens. Pacific Gas and Electric Company Department of Research and Development Report 008.1-94-12, Table 3-2, pp. 3-2 to 3-3.

[] 2005 California Energy Commission (CEC) Title 20 Appliance Efficiency Regulations, CEC 400-2005-012, p. 69.

[] DEER2011 Update Net-To-Gross table  (05/16/2012) http://www.deeresources.com/files/DEER2011/download/DEER2011_NTGR_2012-05-16.xls

[] 2004-2005 Database for Energy Efficiency Resources (DEER) Update Study Final Report, pp. 3-15 to 3-18.