Work Paper WPSDGENRHC1010

Revision 1

San Diego Gas & Electric

Energy Efficiency Engineering

Commercial Evaporator Coil Cleaning

# Document Revision History

| Revision # | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 06/04/2012 | Charles Harmstead/SDG&E | - Adopted from Conservation Services Group WPSDNRHC-EvapCoilClean Rev0. - Updated NTG to DEER 2011 |
| 0.1 | 06/12/2014 | Rocaciano Vega/RMS | INTERNAL REVISION ONLY – no material impact made - Updated to new workpaper template - Added GSIA - Updated EUL_ID, no change to EUL or RUL - Revised Measure Cost language to indicate that cost is from the program due to it being a Direct Install measure |
| 1 | 06/28/2016 | Martin Vu/RMS | - Adopted Evaporator Coil Cleaning Energy Savings values from the May 2, 2013 WORKPAPER DISPOSITION FOR Non-Residential HVAC Rooftop Quality Maintenance California Public Utilities Commission, Energy Division. |

# Section 1. General Measure & Baseline Data

## 1.1  Measure & Delivery Description

### 1.1a Measure Description

Evaporator Coil Cleaning improves heat transfer and efficiency in commercial HVAC units.  Over time, the evaporator coil may accumulate dust and debris which interfere with heat transfer properties and degrade the performance of the AC unit.

Table 1 Measure Names

| Product Code | Measure name |
| --- | --- |
| N/A | Commercial Evaporator Coil Cleaning |

### 1.1b Delivery and Incentive Mechanism

Delivery Mechanism: Financial Support / Direct Install

Program Type/Install: Retrofit (RET)

### 1.1c Measure Requirements

This measure is performed by licensed HVAC contractors as part of a comprehensive AC tune-up service.  Participating Commercial HVAC contractors market the tune-ups to their customers, enabling the program to target hard-to-reach market segments. Website, direct outreach, and program marketing materials distributed through business and trade associations enable greater penetration into targeted market segments. This service is provided to customers at low or no cost, depending on prices set by participating contractors.

## 1.2  DEER Differences Analysis

This measure does not exist in the DEER database.  However, eQUEST 3.54/DEER 2.1 models  were used to establish the baseline energy usages to which the measure savings apply.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain this type of measure. |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

## 1.3  Code Analysis

There are neither federal nor state codes applicable to commercial evaporator coil cleaning.

Table 3 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | N/A | July 1, 2014 |
| Title 20 (2014) | N/A | July 1, 2014 |

California’s Title 24 Building Energy Efficiency Standards include provisions for increasing the efficiency of HVAC replacement units beginning January 1, 2010. The code effects on this measure are minimal and indirect.  Because higher efficiency units are typically more expensive than the prior standard, some customers may be inclined to continue to use their existing units rather than upgrade to the new efficiency standard.  This may serve to increase demand for HVAC tune-ups and the need for the evaporator coil cleaning measure.

## 1.4  Measure Effective Useful Life

The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value, the updated CPUC ED EUL table documentation issued on February 5, 2014, “DEER2014-EUL-table-update_2014-02-05.xlsx”, was consulted. Table 4 below identifies the value/methodology used for the measures in this work paper.

Table 4 EUL Value/Methodology

| EUL_ID | Measure Description | Sector | Bldg Type | Bldg Loc | Basis Type | Version |
| --- | --- | --- | --- | --- | --- | --- |
| HVAC-ClnEvapCoils | Clean Evaporator Coils | Commercial | Any | Any | Rated Years | ExAnte 2013 |

## 1.5  Net-to-Gross Ratios for Different Program Strategies

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 4 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual. Evaporator coil cleaning is performed as HVAC maintenance generally while refrigerant charge adjustments are also performed. The relevant NTGR for this measure is shown in Table 5 below.

Table 5 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | Use Category* | Use SubCategory* | TechGroup* | TechType* | BldgType* | ProgDelivID* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mHVAC-RCA | HVAC Maintenance: Refrigerant Charge Adjustment (RCA); all delivery mechanisms except upstream | Com | Service | Diagnostic | dxAC_equip | SpltSEER | Any | NonUpStrm |

*Denotes that the column is taken from the DEER NTG Table.

## 1.6  Time-of-Use Adjustment Factor

As directed by the CPUC in decision 06-06-063 dated June 29, 2006, time-of-use (TOU) adjustment factors are to be applied for residential A/C and commercial A/C (packaged and split-system direct-expansion cooling) measures only.

Table 6  TOU Summary Table

| Measure | % |
| --- | --- |
| Commercial Evaporator Coil Cleaning | 100 |

*Note:  Check Section 3 if a measure appears to require a non-zero percentage but is assigned zero.  If the load shape is a DEER08 load shape, a TOU of 0 is correct.

# Section 2. Energy Savings & Demand Reduction Calculations

## 2.1  Savings Estimates

The May 2, 2013 Workpaper Disposition for Non-Residential HVAC Rooftop Quality Maintenance

California Public Utilities Commission, Energy Division, herein Disposition, provides a comprehensive list of workpapers that cover all commercial HVAC maintenance measures including WPSDGENRHC1010 Commercial Evaporator Coil Cleaning. The Disposition directs the utilities to revise all unit energy savings (UES) values to use the Commission Staff recommended values obtained from the 20132014-CommercialHVACMaintenance-SavingsValues-April2013-v1-2.xlsx workbook.

Attachment A, of the Disposition, provides background for Refrigerant Charge Adjustment (RCA) Related UES Values.  Attachment A also details the development of approved UES for refrigerant charge adjustment, condenser and evaporator coil cleaning, and air flow adjustment tasks. Savings estimates for workpapers that include refrigerant charge and air-flow adjustment related claims differ significantly in their approaches to determining those claim values.  Rather than examining each workpaper and approach separately, this disposition provides a single set of values based on those provided in the 2011 DEER.

Attachment A indicates that it is important to note that current DEER UES values represent program end goals that could be obtained via a properly functioning maintenance program.  Cooling system efficiency improvements used to generate these savings were obtained by a cadre of skilled technicians focused on maximizing system efficiency improvements.  These same technicians currently serve as master technicians in the ongoing program EM&V effort.  Commission staff believes that service technicians of this level of expertise operate in IOU-sponsored QM efforts, however staff also believes a significant quantity of service technicians do not possess a master’s level of expertise and program benefits suffer as a result.

The general approach in establishing task-based UES values includes the following steps:

- Start with UES values from the 2011 DEER for commercial RCA efforts.  Again, these values are viewed as a realizable goal for all QM efforts.

- Separate those values into task components of charge adjustment, condenser and evaporator coil cleaning and air flow adjustment.

- Provide adjustments to the various components to account for proper application of DEER assumptions and installation rates that represent best estimates of the current state of programs in obtaining DEER values.

Estimation of Non-Charge Related Services Based on DEER Refrigerant Charge Measures

QM workpapers recognize that RCA measure savings associated with DEER estimates include a mix of treatments.  Potential treatments include RCA, condenser and evaporator coil cleaning, air flow adjustments and other fault corrections like removal of non-condensables or line restrictions in the refrigerant system.  Unfortunately, the actual tasks performed that generated those savings were not published, nor were the incremental benefits of various treatments measured.

The approach taken in IOU workpaper development efforts was to assume that efficiency improvements associated with minimal charge adjustments (less than 10% change in charge) were all a result of coil cleaning and air flow adjustments.  While this approach has merit, there are no data to support this assumption. Without better information, staff assumes the benefit of non-charge related coil cleaning and air flow adjustment to equal 25% of DEER charge adjustment UES values, or

Non-Charge Adjustment UES Values = DEER RCA UES values * 0.25                 (1)

There is no known evidence as to the relative impact from the three measures – condenser coil cleaning, evaporator coil cleaning and air flow adjustment - that generate non-charge adjustment savings.  It is generally recognized that typical efficiency improvements associated with condenser coil cleaning are much larger than the other two.  Given a paucity of direct measurements of field conditions, staff recommends the following apportioning of non-charge adjustment savings among the three possible measures:

- Condenser Coil Cleaning – 50% of the total

- Evaporator Coil Cleaning – 25% of the total

- Air Flow Adjustment – 25% of the total.

Note that the total non-charge adjustment UES values for coil cleaning and air flow adjustment apply to all treated units, even those whose threshold charge adjustment would not qualify for RCA savings.  Based on these assumptions, UES values for condenser and evaporator coil cleaning and air flow adjustment are as follows:

Condenser Coil Cleaning UES Values = DEER RCA UES values * 0.125                 (2)

Evaporator Coil Cleaning UES Values = DEER RCA UES values * 0.0625                 (3)

Air Flow Adjustment UES Values = DEER RCA UES values * 0.0625                 (4)

The DEER RCA value multipliers are provided in Cell B2 of the respective UES value tabs for the various treatments in the workbook “20132014-CommercialHVACMaintenance-SavingsValues-April2013-v1-2.xlsx.”  These tabs are Cond Coil Cleaning for condenser coil cleaning UES values, Evap Coil Cleaning for evaporator coil cleaning UES values, and Air Flow Adjustment for air flow adjustment UES values.

The workpaper kWh electric savings and kW peak demand reduction values were updated to reflect the Disposition steps found in Attachment A for the Commercial Evaporator Coil Cleaning measure. The yellow columns in Table 7 designate the prior workpaper values before the Disposition while the green column in Table 7 represents the updated Disposition values.

Because the 2013 Disposition did not include energy savings for the Grocery Store building type, the Disposition provided guidance that the energy savings values for the SCE Small Office building type should be used for SCE’s Grocery Store building type. Since SCE does not have Climate Zone 7, SDG&E conservatively adopted the energy savings values from SCE’s Small Office in Climate Zone 6 because Climate Zone 6 has the lowest energy savings values in California.

Table 7 Updated Disposition Energy Savings and Peak Demand Reduction Values

Table 8.  2013 Disposition Energy Savings Values for SCE’s Grocery Store Building Type

## 2.2  Gross Savings Installation Adjustment

For all the savings discussed above, there is an installation rate applied to values associated with the installation GSIA ID in Error: Reference source not found. The GSIA ID is identified in the ex-ante implementation tables for all programs and measures. The installation rate (IR) is applied to the gross savings calculations using the values associated with the IDs below.

Table 9 Gross Savings Installation Adjustment (GSIA) IDs

| GSIA_ID | Description | Sector | BldgType | UseCategory | TechType |
| --- | --- | --- | --- | --- | --- |
| Com-RCA-SDG | Commercial Refrigerant Charge & Airflow Adjustment; Annual Installation Rate | Res | Any | Service | SpltSEER |

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  See Table 10 for a list of all Building Types and Load Shapes.  See the KEMA report for a more thorough discussion regarding the load shapes for this measure.

Table 10 Building Types and Load Shapes

| Building Type | Load Shape |
| --- | --- |
| Assembly | NON_res:DEER:HVAC_Refrig_Charge |
| Education – Primary School | NON_res:DEER:HVAC_Refrig_Charge |
| Education – Relocatable Classroom | NON_res:DEER:HVAC_Refrig_Charge |
| Education – Secondary School | NON_res:DEER:HVAC_Refrig_Charge |
| Grocery | NON_res:DEER:HVAC_Refrig_Charge |
| Lodging – Hotel (Guest Rooms) | NON_res:DEER:HVAC_Refrig_Charge |
| Manufacturing – Bio/Tech | NON_res:DEER:HVAC_Refrig_Charge |
| Manufacturing – Light Industry | NON_res:DEER:HVAC_Refrig_Charge |
| Health/Medical – Clinic | NON_res:DEER:HVAC_Refrig_Charge |
| Office – Small | NON_res:DEER:HVAC_Refrig_Charge |
| Restaurant – Fast Food | NON_res:DEER:HVAC_Refrig_Charge |
| Restaurant – Sit Down | NON_res:DEER:HVAC_Refrig_Charge |
| Retail – Single Story Large | NON_res:DEER:HVAC_Refrig_Charge |
| Retail – Small | NON_res:DEER:HVAC_Refrig_Charge |
| Storage – Conditioned | NON_res:DEER:HVAC_Refrig_Charge |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

For this measure category, the base case cost is assumed to be zero because these are discretionary modifications (retrofit) to the customers’ existing equipment.  Their alternative is to make no changes to their existing system.

## 4.2 Gross Measure Cost

For Direct Install measures, the cost is obtained through contractor costs and collected by the program.

# Attachments

# References

The eQuest 3.56 Deer 2.1 version of eQuest can be used to recreate the weather sensitive savings reported in the official 2.1 version of the DEER database. 
 http://doe2.com/download/DEER/MAStool/eQUEST-DEERv3-54_Setup.exe

DEER2014 EUL Table Update (02/05/2014) http://www.deeresources.com/files/DEER2013codeUpdate/download/DEER2014-EUL-table-update_2014-02-05.xlsx

DEER2011 Update Net-To-Gross table  (05/16/2012) http://www.deeresources.com/files/DEER2011/download/DEER2011_NTGR_2012-05-16.xls

Energy Efficiency Policy Manual. Version 4.0. California Energy Commission. Updated March 2008. R.06-04-010 DGX/avs. http://docs.cpuc.ca.gov/efile/rulings/80685.pdf

D.06-06-063 Interim Opinion: 2006 Update of Avoided Costs and Related Issues Pertaining To Energy Efficiency Resources http://docs.cpuc.ca.gov/PublishedDocs/WORD_PDF/FINAL_DECISION/57756.PDF

May 2, 2013 Workpaper Disposition for Non-Residential HVAC Rooftop Quality Maintenance

California Public Utilities Commission, Energy Division.

KEMA, JJ Hirsch and Associates, and Itron, Inc. Final Report Load Shape Update. Prepared for the CPUC under contract to PG&E. November 15, 2006.

EUL Value/Methodology

EUL_IDMarketEnduseMeasureHVAC-ClnEvapCoilsNon-ResidentialHVACClean Evaporator Coils

Net-to-Gross Ratio

NTGR_ID*Description*Sector*BldgType*ProgDelivID*NonRes-sAll-mHVAC-RCAHVAC Maintenance: Refrigerant Charge Adjustment (RCA)ComComPreReb

Gross Savings Installation Adjustment (GSIA) IDs

GSIA_IDDescriptionSectorBldgTypeUseCategoryTechTypeCom-RCA-SDGCommercial Refrigerant Charge & Airflow Adjustment; Annual Installation RateResAnyServiceSpltSEER